# T3 - TEARCI

# Julio Filho, N7

# GROUP 3 - DHT22–LCD

C-1 – Temperature Sensor (DHT22): Purpose and Basic Functional Principle

The purpose of the DHT22 sensor is to measure the ambient temperature and humidity of the system.

It works by using an internal temperature and humidity sensing element and sends the measured values as digital data to the microcontroller through a single data pin.

C-2 – LCD with I²C Interface

The LCD display is used to show information to the user, such as temperature values, system status, or messages.

Using I²C instead of parallel wiring has advantages like fewer wires, simpler connections, and saving microcontroller pins, which makes the circuit cleaner and easier to expand.

C-3 – Hysteresis Control Logic

Hysteresis is a small difference between the ON and OFF switching points of a control system.

It is important in temperature control because it prevents the output from switching ON and OFF repeatedly when the temperature is close to the set point, increasing system stability and component life.

C-4 – State Memory Variable

It is necessary to store the output state so the system remembers whether it is currently ON or OFF.

Without storing the state, the output could change too often, causing unstable behavior, especially when hysteresis logic is used.

C-5 – Output Stage (Digital Pin D3)A relay should not be connected directly to a microcontroller pin because the pin cannot supply enough current and the relay can generate high voltage spikes.

An interface stage is required, usually including a transistor, a flyback diode, and sometimes an opto-isolator, to protect the microcontroller.

C-6 – Reliability and Safety

If the sensor fails, the system may read incorrect temperature values, which can lead to overheating or not activating the output when needed.

Simple safety improvements include checking for invalid sensor readings, using default safe states, adding alarms, or implementing a maximum/minimum temperature limit.